# SP Information for Station 1

Read to the HCP: “I am a 19-year-old basketball player who is two weeks post-op acromioclavicular ligament (ACL) and meniscal repair for my right knee. I underwent an allograft tendon graft. I am unable to contract my quadriceps muscle on my own and lack full extension.”

What type of electrical stimulation would you select to help with quadriceps muscle reeducation?”

Following are the parameters the HCP should select for this case, with the primary treatment goal being muscle reeducation.

Neuromuscular Electrical Stimulation (Nmes) Parameters

- Treatment time: 10 to 20 minutes

- Waveform: Russian or functional electrical stimulation (FES)

- Carrier frequency: 2,500 Hz

- Pulse width: 400 µs

- Pulse rate: 30 to 75 pulses per second (pps)

- Intensity: Until a visible muscle contraction is reached

- Duty cycle: 10 seconds on and 50 seconds off because this is the first time the patient is receiving this treatment

- Ramp: Two seconds (the longest option the machine allows)

  - Can decrease as patient gets used to treatment

- Electrode placement: Two electrodes, on either end of the muscle belly

- Patient position

  - Patient long-seated with towel under involved knee

  - Instruct patient to contract their quad with stimulation and push their knee down into the towel

What Are the Physiological Effects?

- Alpha motor neurons are depolarized, and NMES causes the muscle to generate an action potential, resulting in an involuntary contraction

- After several repeated contractions, the central nervous system (CNS) receives and processes afferent feedback from the muscle

- This improves the patient’s proprioception and visual sense of motions, which allows the patient to relearn voluntary muscle contraction and neuromuscular control