# SP Information for Station 6

Read to the HCP: “I am a pole vaulter. I was going up for a vault this morning when my pole broke, and I fell in the box on my left ankle. I was told I sustained a medial ankle sprain. Pain is a 3/10 but I have a lot of swelling around my medial malleolus. Any fractures have been ruled out, and the ankle sprain diagnosis is confirmed. What type of modality would you select to meet the treatment goal of reducing edema?”

Following are the parameters the HCP should select for this case, with the primary treatment goal of reducing acute edema.

High-voltage Pulsed Current (HVPC) Parameters

- Treatment time: 20 to 30 minutes

- Waveform: Monophasic

- Polarity: Negative

- Pulse rate: 120 pps

- Pulse width: 60 to 100 µs

- Intensity: Until the patient feels a sensory tingling but no muscle contraction

- Electrode placement

  - Negative electrode at the area of edema

  - Positive electrode around the medial malleolus

- Patient position

  - Supine or long-seated

  - Involved ankle elevated

What Are the Physiological Effects?

- Negatively charged electrical impulses repel the negatively charged plasma protein from the edema site

- HVPC increases circulation and venous return acting as an edema pump to promote lymphatic drainage